## Distributed Chat System Performance Analysis

### 1. Project Overview

This project involves the development and performance testing of a multithreaded Java WebSocket client. The client is designed to stress-test a chat server deployed on an AWS EC2 t2.micro instance. The primary objective is to evaluate system stability, throughput, and latency under varying levels of concurrency—32, 64, and 128 threads—transmitting a total of 500,000 messages per test.

### 2. Client Architecture & Implementation

Threading Strategy: Utilized a FixedThreadPool to maintain a consistent load on the server throughout the test duration.

Decoupled Communication: Implemented a Producer-Consumer pattern using a LinkedBlockingQueue to decouple message generation from network transmission, allowing sender threads to poll and send messages as fast as the network allowed.

Stability Enhancements:

Broadcast Handling: A silent MessageHandler was added to the client to consume incoming server broadcasts, preventing IllegalStateException during high-load scenarios.

Connection Staggering: Introduced a 50ms delay between thread starts to mitigate the CPU spike caused by simultaneous WebSocket handshakes on the single-core EC2 instance.

Rate Limiting: Implemented a Thread.sleep(1) interval within the sender loop to prevent local buffer overflow and server-side connection resets.

### 3. Performance Results Summary

The following table reflects the data collected during the final phase of experimentation:

| Concurrency (Threads) | Success Rate | Throughput (msg/s) | Mean Latency (ms) | P99 Latency (ms) | Status |
| --- | --- | --- | --- | --- | --- |
| 32 Threads | 100% | 49,850.07 | 0.38 | 0.22 | Optimal Point |
| 64 Threads | 100% | 31,854.38 | 0.58 | 0.83 | Stable |
| 128 Threads | ~66% | ~46,958.47* | 0.47 | 0.89 | Saturation |

*Note: 128-thread test showed high failure rates due to server-side connection resets.

### 4. Analysis and Discussion

A. Identifying the "Sweet Spot" (32 Threads)

The 32-thread configuration yielded the highest stable throughput of 49,850 msg/s. In a resource-constrained environment like the t2.micro (1 vCPU), 32 threads provided enough concurrency to saturate the network without overwhelming the CPU with context-switching overhead.

B. Saturation and Thrashing (128 Threads)

The 128-thread test represented the Saturation Point.

While raw throughput remained high, the system experienced significant instability with 180,375 failures, primarily due to Connection Reset errors. This indicates that the server's TCP backlog or memory limits were exceeded, leading to "thrashing" where the cost of managing connections outweighed the benefit of added concurrency.

C. Little’s Law Validation

Using the 32-thread data, the predicted latency is 32 / 49850 = 0.64ms.

The measured Mean Latency of 0.38ms aligns closely with this prediction, validating that the system was operating at peak efficiency under this load.

### 5. Final Conclusion

The experiments demonstrate that for low-power instances like t2.micro, higher thread counts do not linearly scale performance. The optimal operating point was found at 32 threads, providing the best balance of ultra-low latency (P99 < 1ms) and maximum reliable throughput.